Potato Control System Methodology

The system is divided into two components: distributed control boxes and a central control hub. Each box operates as an independent, compartmentalized control system regulated by an Arduino Uno R4. A Raspberry Pi Model 4 serves as the central hub, collecting data from each box and distributing control parameters.

Each control box consists of several modules, all managed by the Arduino. Temperature data is collected using five Pt100 resistance temperature detectors (RTDs), with four sensors inside the box and one measuring ambient temperature. A NEMA 17 stepper motor, driven by a DRV8825 controller, mechanically adjusts the voltage transformer supplying power to the heaters. Voltage is monitored using a ZMPT101B voltage transformer module. Communication with the control hub is handled via LoRa RYLR998 transceivers.

Inside the algorithm, sensor data is collected every 15 seconds. The target temperature inside the box is the ambient temperature plus a configurable temperature delta. If the difference between the average internal temperature and the target temperature exceeds 1°F, and at least 4 minutes have passed since the last adjustment, the system initiates a voltage correction. Voltage adjustments are proportional: the system changes the transformer output by 3 V per degree of temperature error. Each box transmits instantaneous data every minute and continuously listens for incoming commands from the control hub.

The control hub is hosted by a Raspberry Pi Model 4. The Pi runs a continuous Python process that listens for transmissions from the boxes using a LoRa RYLR998 module. The Pi logs data to both a local PostgreSQL database and an online ThingSpeak channel for redundancy. It also hosts a web server that serves the frontend at potatoheatbox.live. The backend server enables retrieval and download of all relevant data from the boxes. The web interface enables data monitoring, data export, and remote configuration of each box. Users can adjust the temperature delta or schedule automated changes at regular intervals.